The Spectrum Analyzer instrument measures the magnitude of an input signal versus its frequency.
The Spectrum Analyzer uses the oscilloscope channels to acquire input signals. Because of this, all instruments that use the oscilloscope channels will cease acquisition when the Spectrum Analyzer begins sampling.

Figure 1. Spectrum analyzer

1. Menu  […]  1.1 View […]

2. Control

The control toolbar by default only shows the important options. The down/up arrow in the top left corner
shows/hides the other features. This toolbar contains the following:

- Single button: starts a single acquisition.

- Run/Stop button: starts repeated acquisition. While the acquisition is in progress, the Run button
becomes the Stop button.

- Freq. Range: The frequency range options selects the operation range of the Spectrum Analyzer. Having
the auto option selected, the maximum frequency or sampling rate is determined based on the stop or
highest value between start and stop.

- Scale: Selects between Linear and Logarithmic frequency scale.

- Center/Span or Start/Stop: The frequency halfway between the stop and start frequencies is known as
the Center frequency. This is the frequency that is in the middle of the display’s frequency axis. The Span
parameter specifies the range between the start and stop frequencies. Alternatively, the Start and Stop
frequencies can be specified.

- BINs: Adjusts the number of frequency bins.

- Samples: Adjusts the number of time domain and oscilloscope acquisition samples.

- Resolution: Adjusts the frequency resolution bandwidth. The gear drop-down contains the following
options:

- Algorithm:
o FFT: Fast Fourier Transform (quick process) expects a number of input samples (always a power
of 2). It then outputs half of this number in frequency BINs, linear from 0 to half of the sampling
frequency.
o CZT: Bluestein's Chirp-Z Transform expects an arbitrary number of input samples. It then outputs
arbitrary number of frequency BINs, linear from the specified start to stop frequencies. This
allows resolution to increase by interpolation. It allows smooth adjustment of resolution
bandwidth with a flexible number of input samples and output BINs.

- Update: This specifies the time period at which the application will check the oscilloscope device status
and read the acquired data. Increase the time to reduce the update rate.

- Source : Diligent – WaveForms 2015 Reference Manual